# Automated Conveyor Sorting System

# Introduction

Automation plays an important role in modern industries to increase efficiency and decrease manual effort. The conveyor system is used in most of the industries for transporting objects from one place to another. In some cases the objects need to be classified based on some characteristics such as colour or weight.

This project presents the design and development of a  small automated conveyor system that detects the colour and weight of objects and sorts them into different bins. The system uses sensors and a microcontroller to perform the sorting operation automatically.

The goal of this project is to demonstrate the basic concept of industrial automation using simple Mechanical and electronic components.

# Objective

The main objective of project is to design and develop an Automated Conveyor Sorting System that can detect, analyze and sort objects based on their physical properties like colour and weight using sensors and microcontroller

This project aims to integrate various components like a colour sensor,load cell,IR sensor,DC motor and servo motors with an Arduino to create a real-time sorting mechanism.

↪️Specific Objectives

- To detect the presence of objects using an IR sensor

- To measure the weight of objects using a load cell with HX711 amplifier

- To identify the color of objects using a TCS3200 colour sensor

- To control conveyor movement using a DC motor and motor driver (L298N)

- To sort objects into different categories using servo motors

- To develop a complete system using Arduino UNO for control and processing

Components Used :

- Arduino UNO

- L298N Motor Driver

- Buck Converter (LM2596)

- DC Motor

- Servo Motors (2 units)

- IR Sensor

- TCS3200 Colour Sensor

- Load Cell

- HX711 Load Cell Amplifier

- Breadboard

- 12V Power Supply

- Connecting Wires

- 3D Printed Rollers

- Bearings

- Shaft Rod

- MDF Board

- Bread Board

- Rubber Sheet

- 3D Printed Servo Arms

WORKING PRINCIPLE

The automated conveyor sorting system operates by combining the functions of sensors, actuators, and a microcontroller to perform sorting of objects in a systematic manner.

Initially, the conveyor belt is driven by a DC motor, which provides continuous motion to transport objects from one end to another. When an object is placed on the moving conveyor belt, it starts traveling towards the sensing section of the system.

As the object reaches the sensing area, the IR sensor detects the presence of the object and sends a signal to the Arduino.

This detection ensures that the system begins analyzing the object at the correct position. Once the object is detected, the conveyor may momentarily slow down or continue moving steadily while the sensing process takes place.

The load cell placed beneath the sensing platform measures the weight of the object. The small electrical signal generated by the load cell is amplified and converted into digital data using the HX711 amplifier module, which is then read by the Arduino.

Simultaneously, the colour sensor (TCS3200) detects the color of the object by analyzing the light reflected from its surface. The sensor converts the color information into frequency signals, which are processed by the Arduino to determine the specific color of the object.

Based on the combined input from the weight measurement and color detection, the Arduino processes the data and classifies the object into a specific category. After classification, the conveyor continues to move the object towards the sorting section.

At the sorting point, the servo motor rotates its servo arm to a specific angle depending on the category of the object. This movement pushes the object into the appropriate bin or path assigned for that category.

This entire process is repeated continuously for every object placed on the conveyor belt, resulting in an efficient, accurate, and automated sorting system without the need for manual intervention.

Circuit Diagram

The circuit diagram of the automated conveyor sorting system is shown above. It illustrates the connections between the Arduino, sensors, motor driver, and power supply.

All components are connected through a common ground. The buck converter is used to provide a regulated 5V supply to the Arduino and sensors, while the motor driver is powered separately using a 12V supply.

The diagram clearly represents the interfacing of the colour sensor, IR sensor, load cell with HX711 module, servo motors, and DC motor with the Arduino.

Click Here for the Circuit Diagram.

CONNECTIONS

The connections of the automated conveyor sorting system are made by interfacing all components with the Arduino and providing proper power supply through a buck converter and motor driver.

The 12V power supply is given to the L298N motor driver to drive the DC motor. The same 12V supply is also provided to the input of the buck converter.

The buck converter steps down the voltage to 5V, which is used to power the Arduino, sensors, and servo motors.

The DC motor is connected to the output terminals of the L298N motor driver. The control pins IN1 and IN2 of the motor driver are connected to the digital pins of the Arduino to control the direction of the motor.

The two servo motors are connected to the Arduino through digital pins, while their power is supplied from the 5V output of the buck converter. The IR sensor is connected to the Arduino to detect the presence of objects.

The colour sensor is connected to multiple digital pins of the Arduino for color detection, and its output pin is used to send frequency signals to the Arduino. The load cell is connected to the HX711 amplifier module, which is then connected to the Arduino for weight measurement.

All the ground connections of the components are made common to ensure proper functioning of the system.

DETAILED COMPONENT DESCRIPTION

↪️ Arduino UNO

- The Arduino UNO is the main controller of the system. It processes inputs from sensors and controls outputs like motors and servos.

Pins Used:

- 5V → Power input from buck converter

- GND → Common ground

- D2 → IR sensor output

- D3, D5 → L298N motor driver inputs

- D4, D6 → Servo motors

- D7–D10 → Colour sensor (S0–S3)

- D11 → Colour sensor OUT

- D12 → HX711 DT

- D13 → HX711 CLK

Use:

- Acts as the brain of the system.

↪️ L298N Motor Driver

- Used to control the DC motor and drive the conveyor.

Pins:

- 12V → Connected to 12V supply

- GND → Common ground

- 5V → Logic supply (from buck)

- IN1, IN2 → Connected to Arduino (D3, D5)

- OUT1, OUT2 → Connected to DC motor

Use:

- Controls motor direction and movement.

↪️ Buck Converter (LM2596)

- Used to step down voltage from 12V to 5V.

Pins:

- IN+ → 12V positive

- IN− → Ground

- OUT+ → 5V output

- OUT− → Ground

Use:

- Provides stable 5V to Arduino, sensors, and servos.

↪️ DC Motor

- Drives the conveyor belt.

Connection:

- Connected to OUT1 and OUT2 of L298N

Use:

- Moves objects on conveyor.

↪️Servo Motors (2)

- Used to sort objects.

Pins:

- Red → 5V

- Brown → GND

- Orange → Signal (D4, D6)

Use:

- Push objects into bins using a servo arm.

↪️ IR Sensor

- Detects presence of object.

Pins:

- VCC → 5V

- GND → GND

- OUT → Arduino D2

Use:

- Triggers detection when an object arrives.

↪️Colour Sensor (TCS3200)

Pins:

- VCC → 5V

- GND → GND

- S0 → D7

- S1 → D8

- S2 → D9

- S3 → D10

- OUT → D11

- OE → GND

Use:

- Detects color of object.

↪️ Load Cell

Wires:

- Red → E+

- Black → E−

- Green → A+

- White → A−

Use:

- Measures weight of object.

↪️ HX711 Amplifier

Pins:

- VCC / VDD → 5V

- GND → GND

- DT → D12

- CLK → D13

- A+, A− → Load cell

- E+, E− → Load cell

Use:

- Amplifies and converts load cell signal.

↪️ Breadboard

Use:

- Distributes 5V and GND

- Organizes connections

Connection:

- Buck OUT+ → 5V rail

- Buck OUT− → GND rail

↪️ Power Supply (12V)

Connection:

- 12V → L298N motor driver

- 12V → Buck converter input

Use:

- Provides main power for the system.

APPLICATIONS

- Industrial product sorting systems

- Packaging and assembly lines

- Waste segregation systems

- Food processing industries

- Recycling plants

- Smart manufacturing systems

Problems Faced During the Project

1. Unstable Load Cell Readings

One of the major challenges faced during the project.   was obtaining stable and accurate readings from the load cell. Since the load cell produces very small analog signals, the readings were highly sensitive to vibrations, electrical noise, and loose wiring connections. Initially, the measured values fluctuated continuously and sometimes displayed zero even when weight was applied. This issue became more noticeable when the conveyor motor and servo motors were operating simultaneously.

To reduce this problem, multiple software filtering techniques such as averaging and stabilization delays were implemented. The HX711 module was also powered using 3.3V instead of 5V to reduce electrical noise. Additionally, the wiring connections were improved and secured properly.

2. Electrical Noise from Motor and Servos

The DC motor and servo motors generated electrical noise and electromagnetic interference (EMI), which affected the HX711 load cell amplifier. Whenever the conveyor motor stopped or the servos moved, unstable weight readings were observed. The noise also caused sudden fluctuations in sensor outputs.

To overcome this issue, delay intervals were introduced after stopping the motor so that vibrations and electrical disturbances could settle before taking weight measurements. Servo motors were also detached after movement to minimize PWM interference during weight sensing.

3. Incorrect Color Detection

The TCS3200 color sensor initially produced inconsistent color outputs. In several cases, the sensor repeatedly detected the same color even when different colored objects were placed in front of it. Environmental lighting conditions and improper sensor distance also affected the readings.

To improve color detection accuracy, multiple pulse readings were averaged and threshold comparisons were adjusted experimentally. Proper object placement and controlled lighting conditions helped achieve more reliable RGB detection.

4. Servo Motor Stability Issues

The SG90 servo motors did not hold angles firmly and occasionally showed jittering or shaking movements. This mainly occurred due to insufficient power supply and continuous PWM corrections by the servos. The shaking affected the sorting mechanism and caused inaccurate object sweeping.

Instead of using a locking mechanism, a sweeping mechanism was designed in which the servo quickly pushes the object into the required bin and returns to its home position. This reduced the load on the servos and improved overall reliability.

5. Conveyor Belt Alignment Problems

Maintaining proper conveyor belt alignment was another challenge during the mechanical assembly. Improper roller alignment caused uneven belt movement and occasional slipping of objects during transportation. The belt tension also had to be adjusted carefully to ensure smooth operation.

To solve this issue, roller supports and shaft supporters were redesigned and aligned properly. The mechanical structure was strengthened to improve stability during continuous operation.

6. Integration of Multiple Components

Integrating all hardware components into one synchronized system was a challenging task. Individual modules such as the load cell, color sensor, servo motors, and LCD worked correctly when tested separately, but issues appeared when all modules operated together simultaneously.

Careful timing control, sequencing of operations, and debugging were required to achieve stable integration. Delays were introduced between sensing and actuation stages to ensure proper synchronization between hardware and software operations.

7. Calibration Difficulties

The calibration factor of the load cell changed after modifying the power supply configuration and integrating the system with other components. Accurate weight measurement required repeated calibration trials using known weights.

Several experimental tests were conducted to determine a suitable calibration factor and weight threshold for differentiating HIGH and LOW weight objects accurately.

8. Timing and Synchronization Challenges

Proper coordination between conveyor movement, IR detection, color sensing, weight measurement, and servo actuation was critical for smooth operation. Incorrect timing caused premature sensing or inaccurate sorting.

To solve this issue, the complete workflow was redesigned with proper sequencing:

- Conveyor motor runs

- IR sensor detects object

- Motor stops

- Object stabilizes

- Color and weight are measured

- Servo performs sorting action

CONCLUSION

The automated conveyor sorting system successfully demonstrates the integration of sensors, actuators, and a microcontroller to perform sorting operations efficiently. The system is capable of detecting objects, analyzing their weight and color, and sorting them automatically without manual intervention.

This project highlights the practical application of embedded systems in automation and industrial processes. It can be further improved by increasing accuracy, adding more sensors, or implementing advanced control techniques.